|  |  |  |  |  |  |  |  | Technical specification |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Документ nr. Document no. 10HDE00-PB402-0 |  |  |  |  |  | Страница Sheet | 1 |  | от | 7 |
| Проект Project | TPP Contour Global Marisa East 3 |  |  |  |  |  |  |  |  |  |  |  |  | Код Security Index |  |  |  |  |
| Име Title | Delivery of spare parts for replacement fields 4C and 4D the ESP Unit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Система System | AD& |  | Тип документ Document Type |  |  | PВ |  | Дисциплина Discipline |  |  | & |  | Файл File |  |  |  |  |  |
| REV | Описание на ревизиите / Description of Revisions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 03 | FOR TENDER (TR) Първо издание |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 03 | 30.10.2015 | TR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Benishev | ID |  | RY |  |  |  |  | P. Panayotov |  | S.Bodurov |  | S.Bodurov |  |  |
| REV | Дата Date | Обхват Scope |  | Подготвил Prepared by | Сътрудници Co-operations |  |  |  |  |  |  | Проверил Checked by |  | Одобрил Approved by |  | Издал Issued by |  |  |

CONTENTS

1.	Scope of Work	3

2.	General characteristics of the power plant	3

3.	Technical requirements for the delivery	4

4.	Terms of delivery	6

4.1	Schedule	6

4.2	Delivery by the contractor	6

5.	Access in TPP Contour Global Mariza East 3	6

5.1	Working time	6

5.2	Rules for entry into the plant for delivery of goods	6

6.	Wrapping, packaging and assembling	7

7.	Protocols and reference documents	7

# Scope of Work

The scope of work includes the following activities \ supplies:

Delivery of spare parts for replacement the ESP 1 type AOC 130 , fields 4C and 4D.

4C

4D

# General characteristics of the power plant

TPP Contour Global Mariza East 3 is located approximately 60 km southeast of Stara Zagora, 10 km southeast of Galabovo and 2 km north of the village Mednikarovo in Haskovo region near to the open mine Troyanovo 3.

GPS: N–42.143321 и E–26.000856

The reference values ​​of the environmental conditions are:

- Atmospheric pressure: 1004.5 hPa

- Outdoor air temperature (max): 45˚C

- Outdoor air temperature (min): -28.5˚C

- Nominal relative humidity: 73%

- Max. relative humidity: 100%

- Min. relative humidity: 14%

# Technical requirements for the delivery

Spare parts to repair the ESP 1, fields 4C and 4D must be new and meet requirements of the specified technical documentation.

Quantities for supply:

| № | Part Number | Description | QTY |
| --- | --- | --- | --- |
|  | Collecting Surface System |  |  |
| 1 | 08771734-36.00A | COLL PLT HGR BOLT ASSY, CUT THD | 80 |
| 2 | 08772414-203M | ANVIL BEAM, 400MMSPCG X 3 PL | 36 |
| 3 | 08772414-202M | ANVIL BEAM, 400MMSPCG X 2 PL | 4 |
| 4 | 08772737-001M | TOP SUSPENSION BAR ASSEMBLY | 58 |
| 5 | 08611460-462.6B | COLL PL, ESP - 3, 19.0"X462.600" | 464 |
| 6 | 08771741-014 | GUIDE TUBE ASSY, ESP-3 COLL PL | 58 |
| 7 | 08772442-214 | SIDE SPACER CHANNEL, 14 PLATE | 4 |
| 8 | 08772442-215 | SIDE SPACER CHANNEL, 15 PLATE | 4 |
| 9 | 08771015-101 | ADJUSTING BOLT, .75 "DIAX36"LG | 4 |
| 10 | 08772420-214 | CENTER SPACER BAR, 14 PLATE (TOP) | 2 |
| 11 | 08772420-215 | CENTER SPACER BAR, 15 PLATE (TOP) | 2 |
| 12 | 08772441-214 | CENTER SPACER BAR, 14 PLATE (BOTTOM) | 2 |
| 13 | 08772441-215 | CENTER SPACER BAR, 15 PLATE (BOTTOM) | 2 |
| 14 | 08772602-400 | LINK BAR, SIDE SPACER CHANNEL | 4 |
| 15 | 08761300-177 | SNEAK BAFFLE, 120"X1.5"X12.75" | 16 |
| 16 | 08761300-502 | SNEAK BAFFLE, 120"X11.5"X13.25" | 16 |
| 17 | 0FB009020 | FB,.250X2.00X020.00,A36 | 160 |
|  | Collecting Surface Rapping System |  |  |
| 1 | 08641002-201 | EGR ASSY, 240V W/20# PLUNGER | 40 |
| 2 | 07031209 | PIPE-NIP, BLK, 3/4"NPT TO 3/4BSP | 40 |
| 3 | 08661035-018DD | GROUND STRAP,018",1/2-1/2 LUG | 40 |
| 4 | 08661222-13.00 | ADJUSTING BOLT, 198,13"LG | 120 |
| 5 | 08660065-H01 | BOOT SEAL, 198,2X4.5ID,HYPALON | 40 |
| 6 | 08661146 | CLAMP,SS,1-13/16 TO 2-3/4 DIA | 80 |
| 7 | 08661095 | CLAMP, RANGE 4-1/16--5,SS | 40 |
| 8 | 08661117 | SAFETY WIRE, 8" LONG | 120 |
| 9 | 08661137 | GUIDE, CAST, RAPPER SHAFT,2 1/8" | 40 |
| 10 | 08661139-021 | NIPPLE W/FLANGE, 204,21"TALL | 40 |
| 11 | 08661007-017 | SEAL PLATE, 6.5"X6.5"X4.625 ID | 40 |
| 12 | 08661450-003 | RAPPER SHAFT ADAPTER | 40 |
| 13 | 08661353-080.00 | SHAFT, STR-STR,2.0X080.00" | 40 |
|  | High Voltage System |  |  |
| 1 | 08772045-002A | SUPPORT HANGER, HV, 14.25"DIA. | 12 |
| 2 | 08671118-A01 | INS-ALA, 13.63x19.00x24.00H | 12 |
| 3 | 08671117-001 | GSKT, 19.13ODx16.88Dx.25H | 12 |
| 4 | 08671117-002 | GSKT, 13.75ODx11.5IDx.25H | 12 |
| 5 | 07030022 | ROPE-TWSD,CERA,1/4D.2300DG | 100 |
| 6 | 07030129 | ROPE-TWSD,FBR,1/2DIA | 138 |
| 7 | 08772044-001 | SPPT BUSHING MTG PL,,24X24 | 12 |
| 8 | 08771787-059.75 | HANGER FRAME ASSY-HV,WELDED | 8 |
| 9 | 07020262-21.50A | HANGER BOLT W/NUT,21.50,CUT TD | 16 |
| 10 | 08772753-028 | R.D.E. SUPPORT FRAME, 28 GAS ASSAGES | 4 |
| 11 | 08721106-VM012J | RDE, ,2.0X504.000",3",V-PIN,BJ | 449 |
| 12 | 07200063-002 | RDE,PACKAGING SKID | 1 |
| 13 | 08772752-028 | R.D.E. STEADYING FRAME, 28 GAS PASSAGES | 4 |
| 14 | 08771997-009 | HV HGR BOLT ASSY.TEMPORARY | 16 |
| 15 | 08772447-002 | ANTI-SWAY INSULATOR ASSY | 8 |
| 16 | 08771935-002 | BOND JUMPER, RDE SUPPORT FRAME | 2 |
|  | High Voltage rapping System |  |  |
| 1 | 08641002-201 | EGR ASSY, 240V W/20# PLUNGER | 12 |
| 2 | 07031209 | PIPE-NIP, BLK, 3/4"NPT TO 3/4 BSP | 12 |
| 3 | 08661035-018DD | GROUND STRAP, 018", 1/2-1/2 LUG | 12 |
| 4 | 08661222-13.00 | ADJUSTING BOLT, 198, 13"LG | 36 |
| 5 | 08660065-H01 | BOOT SEAL, 198, 2x4.5ID, HYPALON | 12 |
| 6 | 08661146 | CLAMP, SS, 1-13/16 TO 2-3/4 DIA | 24 |
| 7 | 08661095 | CLAMP, RANGE 4-1/16--5, SS | 12 |
| 8 | 08661117 | SAFETY WIRE, 8" LONG | 36 |
| 9 | 08661137 | GUIDE, CAST, RAPPER SHAFT, 2 1/8" | 12 |
| 10 | 08661139-009 | NIPPLE W/FLANGE, 204,9" | 12 |
| 11 | 08661007-017 | SEAL PLATE, 6.5"x6.5"x4.625 ID | 12 |
| 12 | 08661450-003 | RAPPER SHAFT ADAPTER | 12 |
| 13 | 08661125-023.00 | SHAFT, STR-TPR, 2.0x023.000" | 12 |
| 14 | 08660106-000 | ADAPTER, DBL-TPR, 8.5"LG, 2"TO2" | 24 |
| 15 | 08661024-036.00 | SHAFT, DBL-TPR, G-10, 2x36 | 12 |
| 16 | 08661125-071.25 | SHAFT, STR-TPR, 2.0x071.250" | 12 |
| 17 | 08661034-001 | SEAL ASSY, RPPR ROD, 831155-L | 12 |
| 18 | 07040007 | SEAL-EPOXY, DEVCON, 14250, 1OZ | 4 |
|  | High Voltage System - Additional spare parts |  |  |
| 1 | 08671118-A01 | INS-ALA, 13.63x19.00x24.00H | 12 |
| 2 | 08772680-202 | INSULATOR RING HEATER ASSY | 6 |

# Terms of delivery

## Schedule

Delivery will depend on our maintenance schedule for 2016 year.

Delivery must be in the warehouse ConturGlobal not later than 30.04.2016 year.

## Delivery by the contractor

In the case of delivery of goods directly by the contractor, the date and time of the delivery must be notified in advance as at the warehouse as well to the person responsible for the technical control. The goods cannot be accepted at the warehouse if it has not been passed the inspection and the protocol for technical control has not been completed. If the goods meet the request's clauses, acceptance protocol is completed, the goods is received and brought to the warehouse and acceptance protocol and protocol for technical control is passed in accounting department. If it does not meet the specified parameters in the request, it is returned to the supplier.

# Access in TPP Contour Global Mariza East 3

## Working time

The working time at the plant is 24/7. The working time of the warehouse is from Monday to Friday from 8:30 to 15:00. Access is strictly forbidden to the people of the external companies in nonworking hours of the regular shift - before 7:30 am and after 16 pm from Monday to Friday and on Saturday and Sunday.

## Rules for entry into the plant for delivery of goods

- In the case of delivery of letters, packages or small volume of goods that can be brought by hand, they shall be transmitted to main gate to the guard who takes care to deliver them to the recipient. If is necessary entry of the person who brought the goods, after presentation of an identity card, will be issued a temporary pass, through which he can entry to the territory of the plant and to deliver the goods on the recipient or the warehouse.

- In the case of delivery of the goods with vehicle the procedure is: The driver presents payment documents, specifications, supply contracts and other documents certifying that the goods come because of request of CGMI3 or CGOB. А document for entry must be filled on the gate, after that must be signed by the Head of Health, Safety and Security.

- Vehicles with unwashed tires or loose cargo are not allowed inside or outside the plant.

- During loading or unloading of the cargo the driver must use PPE that are required for work on the territory of the TPP - helmet, safety shoes and appropriate working clothes.

- Drivers of vehicles carrying volatile or dispersants are required to put covers on the body of the vehicle.

- Drivers of vehicles on the territory of the plant must drive, stop and park, so that don’t interfere with the movement of other vehicles / fire truck, ambulance, service vehicles / or people.

- After completion of loading or unloading operations vehicles must immediately leave the territory of the plant.

- It is absolutely prohibited import or export of alcohol / including beer /, drugs or other psychotropic substances and access to the territory of TPP of the persons that had used such substances.

- It is absolutely forbidden the import of firearms, gas and blank weapons, explosive materials and Powder actuated tools.

# Wrapping, packaging and assembling

The goods must be delivered packed separately by type of product in standard packaging by the manufacturer.

# Protocols and reference documents

The goods shall be accompanied by the following documents:

- Delivery and acceptance protocol

- Quality Certificate from the manufacturer

- Warranty Card

- Instructions for use from the manufacturer

- Bill of lading and packing list for the transmission of goods or other

- Certificates of quality of the materials.