|  |  |  |  |  |  |  |  | BILL OF QUANTITY Bill of Quantity |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Документ nr. Document no. 90HTA00-PC401 |  |  |  |  |  | Страница Sheet |  | 1 | от of | 7 |
| Проект Project |  | CONTOURGLOBAL MARITSA EAST 3 AD TPP |  |  |  |  |  |  |  |  |  |  |  | Code Security Index |  |  |  |  |
| Име Title |  | Delivery of spare parts and Service maintenance for gas analyzing stations MCA04, Ultramat/Oxymat 6, Ultramat6, dust concentration measuring device PFM97 and system DURAG, installed in FGD in CONTOURGLOBAL MARITSA EAST 3 AD TPP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Система System |  | 90HTA |  | Тип документ Document Type |  |  | PC |  | Дисциплина Discipline |  | & |  | Файл File | 90HTA00-PC401.doc |  |  |  |  |
| REV | Описание на ревизиите / Description of Revisions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 01 | Tender procedure (for tender) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 01 | 27.05.2015 |  | TR |  | Todor Kolev | Georgy Avdjiev |  |  |  |  |  | Plamen Panayotov |  | Sergey Bodurov |  |  |  |  |
| Rev. | Date: |  | Range |  | Prepared by: | Cooperation |  |  |  |  |  | Checked by: |  | Approved by: | Issued |  |  |  |

# Subject:

Service maintenance for gas analyzing stations MCA04,   Ultramat/Oxymat 6, Ultramat 6, dust concentration measuring device PFM97 and system DURAG, installed in FGD, is carried out once a year.

- Works

-  Check and replacement of consumables, defective parts and units, calibration and integral serviceability and correct operation check of gas analyzer MCA04.

-  Check and replacement of consumables, defective parts and units, calibration and integral serviceability and correct operation check of gas analyzer Ultramat/Oxymat 6.

-  Check and replacement of consumables, defective parts and units, calibration and integral serviceability and correct operation check of gas analyzer Ultramat 6.

-  Check and replacement of consumables, defective parts and units, calibration and integral serviceability and correct operation check of dust concentration measuring device PFM97.

-  Check on serviceability of information system DURAG functions - analog and digital input-output, software – complete check.

# BILL OF QUANTITY

A price has been specified for each item, based on tools, equipment, daily costs, transport, shifts and everything else necessary so that well-done works are provided to the Owner as described and requested in the technical specification. The shown quantities for Items 1-6 are for one year. The quantities in Item 7 are for one time delivery.

| Item | Description | Unit | Unit price, BGN | Quantity | Total, BGN |
| --- | --- | --- | --- | --- | --- |
| 1 | Dust concentration measuring device PFM 97 ED, outlet FGD - 12/34HTA11CQ006 |  |  |  |  |
| 1.1 | Visual inspection on the measuring system, fixing air and gas leaks. | nos. |  | 2 |  |
| 1.2 | Check on the measurement of the differential pressure in service mode of operation. | nos. |  | 2 |  |
| 1.3 | Calibration in Zero and Reference point | nos. |  | 2 |  |
| 1.4 | Check on the zero point with switched off blowdown (only at blowdown regime) | nos. |  | 2 |  |
| 1.5 | Check on reliability of the values of F and Fd | nos. |  | 2 |  |
| 1.6 | Check on the temperature of the measuring cell and the diluting air | nos. |  | 2 |  |
| 1.7 | Check on the suction opening of the air blower. | nos. |  | 2 |  |
| 1.8 | Cleaning the ejector | nos. |  | 2 |  |
| 1.9 | Check on the filter for ventilation of the samples and replacement, if necessary | nos. |  | 2 |  |
| 1.10 | Check on the filter for the blowing and if necessary - replacement | nos. |  | 2 |  |
| 1.11 | Overall cleaning of the instrument | nos. |  | 2 |  |
| 1.12 | Inspection of the operation of the instrument (check and analysis on the chronological order for errors and entries in DCS) | nos. |  | 2 |  |
| 1.13 | Overall cleaning of the probe | nos. |  | 2 |  |
| 1.14 | Cleaning with special brushes (ramrod), internally , together with the piping of the prove (sampling point, flexible sampling pipes, taps and so on) | nos. |  | 2 |  |
| 1.15 | Cleaning of the measuring cell | nos. |  | 2 |  |
| 1.16 | Replacement of all bolts and fittings for the sampling head | nos. |  | 2 |  |
| 2 | Gas analyzer inlet FGD - 12/34HTA11CQ001-A01/A02 |  |  |  |  |
| 2.1 | Ultramat 6 - SO2/Oxymat 6 - O2 | Pcs. |  | 4 |  |
| 2.1.1 | Calibration in Zero and end points of all signals | nos. |  | 4 |  |
| 2.1.1 | Replacement of measuring cell filter | nos. |  | 4 |  |
| 2.1.3 | Replacement and cleaning of the route of the sample of gas | nos. |  | 4 |  |
| 2.1.4 | General check of the electrical connections and filters on the panel | nos. |  | 4 |  |
| 2.2 | Sampling probe SP 210 |  |  |  |  |
| 2.2.1 | Check on the thermostat and the temperature controller for SP210-H operation | nos. |  | 4 |  |
| 2.2.2 | Check on the instrument functioning | nos. |  | 4 |  |
| 2.2.3 | Check and if necessary replacement of the filter component SP-2K | nos. |  | 4 |  |
| 2.2.4 | Replacement of fittings Viton (30) | nos. |  | 4 |  |
| 2.2.5 | Replacement of the fittings of the cover SP-210-H | nos. |  | 4 |  |
| 2.2.6 | Replacement О-ring Viton | nos. |  | 4 |  |
| 2.2.7 | Set of О-rings for SP210-H - replacement | nos. |  | 4 |  |
| 2.2.8 | Replacement of the flange fittings DN66 PN6 | nos. |  | 4 |  |
| 2.3 | Electric Gas Cooler EC-30/FD |  |  | 4 |  |
| 2.3.1 | Check on the functioning of the device (3 way valves, cooling, regime) | nos. |  | 4 |  |
| 2.3.2 | Washing the piping | nos. |  | 4 |  |
| 2.3.3 | Checks of leaks or blockages in the system | nos. |  | 4 |  |
| 2.3.4 | Check on the state of peristaltic pumps and if necessary - replacement of the worn out parts | nos. |  | 20 |  |
| 2.3.5 | Check on the level and the operation of the Humidifier FP-BF | nos. |  | 4 |  |
| 2.3.6 | Check on the membrane pump and replacement of the worn out parts (diaphragms, valves...) | nos. |  | 4 |  |
| 2.3.7 | Replacement of the flexible tube of the peristaltic pumps | nos. |  | 20 |  |
| 3 | Gas analyzer inlet FGD 3 4 - 34HTA11CQ003/4 |  |  |  |  |
| 3.1 | Ultramat 6 –NO,CO | nos. |  | 2 |  |
| 3.1.1 | Calibration in Zero and end point of all signals | nos. |  | 4 |  |
| 3.1.2 | Replacement of measuring cell filter | nos. |  | 4 |  |
| 3.1.3 | Replacement and cleaning of the route of the sample of gas | nos. |  | 2 |  |
| 3.1.4 | General check of the electrical connections and filters on gas analyzer. | nos. |  | 2 |  |
| 4 | Gas analyzer MCA04 - output FGD - 12/34HNE10CQ001 MCA04 - SO2, NO, CO, CO2, H2O |  |  |  |  |
| 4.1 | Cleaning and replacement of the filters in the panels | nos. |  | 4 |  |
| 4.2 | Blowdown, flushing of the piping route of the sample of gas | nos. |  | 4 |  |
| 4.3 | Check and cleaning of the calibration gases piping system | nos. |  | 4 |  |
| 4.4 | Replacement of the sampling lance filter | nos. |  | 4 |  |
| 4.5 | Replacement of the sealings of the sampling lance | nos. |  | 4 |  |
| 4.6 | Overall check and tide the electrical connections | nos. |  | 4 |  |
| 4.7 | Calibration of the instrument with standard gases (zero point, linearity, scale and so on ) and issuing a protocol | nos. |  | 4 |  |
| 4.8 | Checking energy level of the measuring lasers and issuing a statement | nos. |  | 4 |  |
| 4.9 | Check on the system tightness | nos. |  | 4 |  |
| 4.10 | Check for corrosion of the parts exposed to gases | nos. |  | 4 |  |
| 4.11 | Replacement of the sampling pump diaphragm | nos. |  | 4 |  |
| 4.12 | Replacement of measuring cell filter | nos. |  | 4 |  |
| 4.13 | Replacement of the inlet elbow of the measuring cell | nos. |  | 4 |  |
| 4.14 | Replacement of non-return valve, filter component, sealings of the cell. | nos. |  | 4 |  |
| 4.15 | Check on the history for alarms and events and analysis on them. | nos. |  | 4 |  |
| 4.16 | Replacement of the PLC battery | nos. |  | 4 |  |
| 5 | DURAG system - 00HNE10GH001 |  |  |  |  |
| 5.1 | Diagnostic efficiency of the input and output modules, power supply, software ... | nos. |  | 1 |  |
| 5.2 | Check on the electrical components and connections of DURAG panel. | nos. |  | 1 |  |
| 6. | Work not foreseen in the sections above. | man-hours |  | 50 |  |
| 7. | Delivery of spare parts |  |  |  |  |
| 7.1 | Cell filter ETL 850 | nos. |  | 8 |  |
| 7.2 | Sample gas cell ETL 840 | nos. |  | 8 |  |
| 7.3 | Membrane set for pump ETL 515 | nos. |  | 8 |  |
| 7.4 | Filter element ETL 113 | nos. |  | 12 |  |
| 7.5 | Filter element 0,1 GF MTC 2564 ETL 91 | nos. |  | 16 |  |
| 7.6 | Check valve ETL 827 | nos. |  | 12 |  |
| 7.7 | Connection ETL 852 | nos. |  | 12 |  |
| 7.8 | Fitting for pump 6mm ETL 839 | nos. |  | 8 |  |
| 7.9 | Elbow 90° 6 to 8mm ETL 829 | nos. |  | 8 |  |
| 7.10 | Sealing set for pump N87 TTE ETL 125 | nos. |  | 8 |  |
| 7.11 | Filter element SP 2-K ETL 32 | nos. |  | 12 |  |
| 7.12 | O-ring set ETL33 | nos. |  | 12 |  |
| 7.13 | Filter mat ETL 7627 | nos. |  | 8 |  |
| 7.14 | Filter security ETL 21350 | nos. |  | 12 |  |
| 7.15 | Display MCA04 ETL 577 | nos. |  | 2 |  |
| 7.16 | Hose set for condensate pump ETL 37 | nos. |  | 16 |  |
| 7.17 | Peristaltic pump wheel ETL 36 | nos. |  | 12 |  |
| 7.18 | Sampling tube SP210, 1m ETL 8211 | man-hours |  | 4 |  |
| 7.19 | element for pump ETL 14 | nos. |  | 4 |  |
| 7.20 | Flow meter DK800/PV/K1 for poz. Y02 | nos. |  | 2 |  |
| 7.21 | Probe sampling set for PFM97D | nos. |  | 1 |  |
| 7.22 | Control unit for PFM97D | nos. |  | 1 |  |